"""

rpi_conveyor.py

===============

Conveyor System - Raspberry Pi RS232 communication code

MFC(PC) <-> Raspberry Pi

Hardware:

- RS232 TX/RX  : GPIO14(TX), GPIO15(RX)  via MAX3232 level converter

- DHT11 sensor : GPIO26 (BCM) - 3pin module

- DC Motor IN1 : GPIO12 (BCM) - PWM (speed)

- DC Motor IN2 : GPIO13 (BCM) - Direction fixed LOW

- Stepper IN1  : GPIO17 (BCM)

- Stepper IN2  : GPIO18 (BCM)

- Stepper IN3  : GPIO27 (BCM)

- Stepper IN4  : GPIO22 (BCM)

Control mapping:

Conveyor ON/OFF  → DC 모터 ON/OFF

Speed 1/2/3      → DC 듀티 50/75/100%

Emergency Stop   → DC 모터 정지

Fan ON/OFF       → 스텝모터 ON/OFF

DC Motor direction (바꾸려면 두 줄 swap):

DC_PWM_PIN = 12  ↔  13

DC_DIR_PIN = 13  ↔  12

Run:

sudo python3 rpi_conveyor.py

"""

import serial

import time

import threading

import lgpio

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

# Protocol constants

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

PACKET_SIZE = 9

STX = 0x02

ETX = 0x03

IDX_STX  = 0

IDX_CMD  = 1

IDX_LEN  = 2

IDX_TEMP = 3

IDX_HUMI = 4

IDX_ULTR = 5

IDX_MOTR = 6

IDX_CS   = 7

IDX_ETX  = 8

CMD_STATUS_REPORT    = 0x10

CMD_CONVEYOR_CONTROL = 0x20

CMD_SPEED_SET        = 0x21

CMD_FAN_CONTROL      = 0x22

CMD_EMG_STOP         = 0x23

CMD_ACK  = 0x70

CMD_NACK = 0x71

MOTOR_OFF     = 0x00

MOTOR_SPEED_1 = 0x01

MOTOR_SPEED_2 = 0x02

MOTOR_SPEED_3 = 0x03

MOTOR_EMG     = 0xFF

CTRL_OFF = 0x00

CTRL_ON  = 0x01

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

# GPIO settings

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

SERIAL_PORT = "/dev/ttyS0"

BAUD_RATE   = 9600

DHT_PIN = 26

DC_PWM_PIN = 12

DC_DIR_PIN = 13

STEP_IN1 = 17

STEP_IN2 = 18

STEP_IN3 = 27

STEP_IN4 = 22

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

# Config

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

DC_PWM_FREQ = 1000

DC_SPEED_DUTY = {

MOTOR_SPEED_1: 50,

MOTOR_SPEED_2: 75,

MOTOR_SPEED_3: 100,

}

STEP_SEQUENCE = [

[1, 0, 1, 0],

[0, 1, 1, 0],

[0, 1, 0, 1],

[1, 0, 0, 1],

]

STEP_DELAY = 0.005

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

# Helper

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

def gpio_safe_free(h, gpio):

try:

lgpio.gpio_free(h, gpio)

except lgpio.error:

pass

def gpio_safe_claim_output(h, gpio, level=0):

gpio_safe_free(h, gpio)

lgpio.gpio_claim_output(h, gpio, level)

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

# Packet utilities

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

def calc_cs(pkt):

return (pkt[IDX_CMD] ^ pkt[IDX_LEN] ^

pkt[IDX_TEMP] ^ pkt[IDX_HUMI] ^

pkt[IDX_ULTR] ^ pkt[IDX_MOTR]) & 0xFF

def build_packet(cmd, temp_byte, humi_byte, ultr_byte, motr):

pkt = [0] * PACKET_SIZE

pkt[IDX_STX]  = STX

pkt[IDX_CMD]  = cmd        & 0xFF

pkt[IDX_LEN]  = 5

pkt[IDX_TEMP] = temp_byte  & 0xFF

pkt[IDX_HUMI] = humi_byte  & 0xFF

pkt[IDX_ULTR] = ultr_byte  & 0xFF

pkt[IDX_MOTR] = motr       & 0xFF

pkt[IDX_CS]   = calc_cs(pkt)

pkt[IDX_ETX]  = ETX

return bytes(pkt)

def validate_packet(data):

if len(data) != PACKET_SIZE:  return False

if data[IDX_STX] != STX:     return False

if data[IDX_ETX] != ETX:     return False

cs = (data[IDX_CMD] ^ data[IDX_LEN] ^

data[IDX_TEMP] ^ data[IDX_HUMI] ^

data[IDX_ULTR] ^ data[IDX_MOTR]) & 0xFF

return data[IDX_CS] == cs

def send_ack(ser, success):

cmd = CMD_ACK if success else CMD_NACK

ser.write(build_packet(cmd, 0, 0, 0, 0))

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

# DC Motor - conveyor belt (PWM)

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

class DCMotor:

def __init__(self, h, pwm_pin, dir_pin):

self._h        = h

self._pwm_pin  = pwm_pin

self._dir_pin  = dir_pin

self._speed    = MOTOR_OFF

self._duty     = 0

gpio_safe_claim_output(self._h, self._dir_pin, 0)

gpio_safe_claim_output(self._h, self._pwm_pin, 0)

def _force_stop(self):

"""PWM 끄고 핀을 강제 LOW — 확실한 정지"""

lgpio.tx_pwm(self._h, self._pwm_pin, 0, 0)

time.sleep(0.01)

gpio_safe_claim_output(self._h, self._pwm_pin, 0)

lgpio.gpio_write(self._h, self._dir_pin, 0)

def set_speed(self, speed):

if speed == MOTOR_EMG or speed == MOTOR_OFF:

self._speed = MOTOR_OFF

self._duty  = 0

self._force_stop()

else:

self._speed = speed

self._duty  = DC_SPEED_DUTY.get(speed, 50)

lgpio.tx_pwm(self._h, self._pwm_pin, DC_PWM_FREQ, self._duty)

def get_speed(self):  return self._speed

def get_duty(self):   return self._duty

def cleanup(self):

self._force_stop()

gpio_safe_free(self._h, self._pwm_pin)

gpio_safe_free(self._h, self._dir_pin)

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

# Stepper Motor - fan (ON/OFF only)

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

class StepMotor:

def __init__(self, h, pins):

self._h          = h

self._pins       = pins

self._running    = False

self._step_idx   = 0

self._lock       = threading.Lock()

self._stop_event = threading.Event()

self._thread     = threading.Thread(target=self._run, daemon=True)

for pin in self._pins:

gpio_safe_claim_output(self._h, pin, 0)

self._thread.start()

def _set_coils(self, pattern):

for pin, val in zip(self._pins, pattern):

lgpio.gpio_write(self._h, pin, val)

def _all_off(self):

for pin in self._pins:

lgpio.gpio_write(self._h, pin, 0)

def _run(self):

while not self._stop_event.is_set():

with self._lock:

running = self._running

if not running:

self._all_off()

time.sleep(0.05)

continue

self._set_coils(STEP_SEQUENCE[self._step_idx])

self._step_idx = (self._step_idx + 1) % len(STEP_SEQUENCE)

time.sleep(STEP_DELAY)

def set_on(self, on):

with self._lock:

self._running = on

def is_on(self):

with self._lock:

return self._running

def cleanup(self):

self._stop_event.set()

self._thread.join(timeout=2.0)

self._all_off()

for pin in self._pins:

gpio_safe_free(self._h, pin)

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

# DHT11 reader

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

class DHT11Reader:

def __init__(self, h, gpio_num):

self._h    = h

self._gpio = gpio_num

self._last_temp = 0.0

self._last_humi = 0.0

def _read_raw(self):

g = self._gpio

h = self._h

edges = []

def on_edge(chip, gpio, level, tick):

edges.append((level, tick))

gpio_safe_claim_output(h, g, 1)

lgpio.gpio_write(h, g, 0)

time.sleep(0.020)

lgpio.gpio_write(h, g, 1)

time.sleep(0.00002)

gpio_safe_free(h, g)

lgpio.gpio_claim_alert(h, g, lgpio.BOTH_EDGES, lgpio.SET_PULL_UP)

cb = lgpio.callback(h, g, lgpio.BOTH_EDGES, on_edge)

time.sleep(0.1)

cb.cancel()

gpio_safe_free(h, g)

return edges

def _parse(self, edges):

high_durations = []

for i in range(len(edges) - 1):

if edges[i][0] == 1 and edges[i + 1][0] == 0:

dur = edges[i + 1][1] - edges[i][1]

if dur < 0: dur += (1 << 64)

high_durations.append(dur)

if len(high_durations) < 38:

return None, None

data_pulses = high_durations[-40:] if len(high_durations) >= 40 else high_durations

threshold = sum(data_pulses) / len(data_pulses)

bits = [1 if dur > threshold else 0 for dur in data_pulses]

while len(bits) < 40: bits.insert(0, 0)

bits = bits[-40:]

data = []

byte = 0

for i, b in enumerate(bits):

byte = (byte << 1) | b

if (i + 1) % 8 == 0:

data.append(byte)

byte = 0

if len(data) != 5: return None, None

if ((data[0] + data[1] + data[2] + data[3]) & 0xFF) != data[4]:

return None, None

humi = data[0] + data[1] * 0.1

temp = data[2] + (data[3] & 0x7F) * 0.1

if data[3] & 0x80: temp = -temp

return temp, humi

def read(self):

for attempt in range(5):

try:

edges = self._read_raw()

if len(edges) < 2:

time.sleep(1.0); continue

t, h = self._parse(edges)

if t is not None and h is not None:

self._last_temp = t

self._last_humi = h

return self._last_temp, self._last_humi

else:

print(f"[DHT11] Parse failed (edges={len(edges)}, attempt {attempt+1})")

except Exception as e:

print(f"[DHT11] Error: {e}")

time.sleep(1.0)

print("[DHT11] Read failed, using last value")

return self._last_temp, self._last_humi

def cleanup(self):

gpio_safe_free(self._h, self._gpio)

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

# Main

# ━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

def main():

h = lgpio.gpiochip_open(0)

dht     = DHT11Reader(h, DHT_PIN)

dc      = DCMotor(h, DC_PWM_PIN, DC_DIR_PIN)

stepper = StepMotor(h, [STEP_IN1, STEP_IN2, STEP_IN3, STEP_IN4])

try:

ser = serial.Serial(

port=SERIAL_PORT, baudrate=BAUD_RATE,

bytesize=serial.EIGHTBITS, parity=serial.PARITY_NONE,

stopbits=serial.STOPBITS_ONE, timeout=0.1

)

print(f"[SERIAL] {SERIAL_PORT} opened ({BAUD_RATE} bps)")

except serial.SerialException as e:

print(f"[ERROR] Serial: {e}")

dht.cleanup(); dc.cleanup(); stepper.cleanup()

lgpio.gpiochip_close(h)

return

last_report = 0.0

print("[SYSTEM] Started")

print(f"[SYSTEM] DC Motor(conveyor): PWM=GPIO{DC_PWM_PIN}, DIR=GPIO{DC_DIR_PIN}")

print(f"[SYSTEM] Stepper(fan): GPIO {STEP_IN1}/{STEP_IN2}/{STEP_IN3}/{STEP_IN4}")

print(f"[SYSTEM] DC duty: Speed1={DC_SPEED_DUTY[1]}% Speed2={DC_SPEED_DUTY[2]}% Speed3={DC_SPEED_DUTY[3]}%")

try:

while True:

now = time.time()

if now - last_report >= 2.0:

temp, humi = dht.read()

cur_speed = dc.get_speed()

temp_int = int(temp) & 0xFF

humi_int = int(humi) & 0xFF

temp_dec = int(round(temp * 10)) % 10

humi_dec = int(round(humi * 10)) % 10

ultr_packed = ((temp_dec & 0x0F) << 4) | (humi_dec & 0x0F)

pkt = build_packet(CMD_STATUS_REPORT,

temp_int, humi_int, ultr_packed, cur_speed)

ser.write(pkt)

fan_str = "ON" if stepper.is_on() else "OFF"

print(f"[TX] Temp={temp:.1f}C Humi={humi:.1f}% "

f"DC={cur_speed}({dc.get_duty()}%) Fan={fan_str}")

last_report = now

if ser.in_waiting >= PACKET_SIZE:

raw = ser.read(PACKET_SIZE)

if not validate_packet(raw):

print(f"[RX] Packet error: {raw.hex().upper()}")

send_ack(ser, False)

continue

cmd  = raw[IDX_CMD]

data = raw[IDX_MOTR]

print(f"[RX] CMD=0x{cmd:02X} DATA=0x{data:02X}")

# ── Conveyor ON/OFF → DC motor ──

if cmd == CMD_CONVEYOR_CONTROL:

if data == CTRL_ON:

dc.set_speed(MOTOR_SPEED_1)

print(f"[DC] Conveyor ON (duty={dc.get_duty()}%)")

else:

dc.set_speed(MOTOR_OFF)

print("[DC] Conveyor OFF -> STOPPED")

send_ack(ser, True)

# ── Speed 1/2/3 → DC motor duty ──

elif cmd == CMD_SPEED_SET:

if data in (MOTOR_SPEED_1, MOTOR_SPEED_2, MOTOR_SPEED_3):

dc.set_speed(data)

print(f"[DC] Speed {data} (duty={dc.get_duty()}%)")

send_ack(ser, True)

else:

print(f"[DC] Invalid speed: 0x{data:02X}")

send_ack(ser, False)

# ── Fan ON/OFF → Stepper motor ──

elif cmd == CMD_FAN_CONTROL:

if data == CTRL_ON:

stepper.set_on(True)

print("[STEPPER] Fan ON")

else:

stepper.set_on(False)

print("[STEPPER] Fan OFF")

send_ack(ser, True)

# ── Emergency Stop → DC motor only ──

elif cmd == CMD_EMG_STOP:

dc.set_speed(MOTOR_EMG)

print("[DC] *** EMERGENCY STOP *** -> STOPPED")

send_ack(ser, True)

else:

print(f"[RX] Unknown: 0x{cmd:02X}")

send_ack(ser, False)

time.sleep(0.05)

except KeyboardInterrupt:

print("\n[SYSTEM] Shutdown (Ctrl+C)")

finally:

dc.set_speed(MOTOR_OFF)

stepper.set_on(False)

dc.cleanup()

stepper.cleanup()

dht.cleanup()

ser.close()

lgpio.gpiochip_close(h)

print("[SYSTEM] Cleanup complete.")

if __name__ == "__main__":

main()